###### CHANGE HISTORY:

| Document Iteration | Description of Change |
| --- | --- |
| 00 | Initial Release |

###### PROJECT INFORMATION:

| Product / Project Name | Project Manager | TE Lead |
| --- | --- | --- |
| Metering Syringe Valve Redesign | Peter Kirby | Tyler Modelski / Richard Davis |

# Introduction

  - This Test Equipment Design Plan under the authority of Waters’ management to detail the testing approach for the Metering Syringe Valve (P/N 289010714 and 289012763) based on the description of its operation and functions as detailed in the its Performance Specification (735004417). This document has been written in accordance with the Test Engineering Policy Management Procedure [730001833].

  - SCOPE

The scope of this Test Equipment Design Plan is to provide manufacturing test capability for the Metering Syringe Valve (P/N 289010714 and 289012763). The tester will be located in the Milford, MA White Room.  The type of test equipment required will fall under the category of Sub-Assembly.   Data Collection will be implemented.  Defining the scope of this Test Equipment Design Plan will provide the necessary objectives, schedule, budget, requirements, qualification, and deliverables required for releasing the Metering Syringe Valve Leak Test.

###### PROPOSED SCHEDULE:

###### The schedule to complete this project can be broken down into several phases. Initial concept, lab experimentation, parts procurement, tester build, software development, debug, test qualification, documentation, and training.  The following table lists these phases with an approximate time, considering the risks and contingencies of section 8.0.

| Tasks | Provider | Dependencies | Time(wks) |
| --- | --- | --- | --- |
| Initial Concept / Feasibility | TE | Equipment / Software | 4 |
| Part procurement | Internal, external | Funding/Availability | 2 |
| Tester build | TE Technicians | Tech. schedule | 2 |
| Software development | TE | Equipment, TE schedule | 6 |
| Debug | TE | HW & SW complete, UUT | 2 |
| Qualification per 730001835 | TE | Available UUTs | 2 |
| Training Operators | TE | Available Ops/UUTs | 1 |
| Test Specification Procedure | TE |  | 2 |
| Calibration and Preventive Maintenance Procedure | TE |  | 2 |

| Planned Initial Product Shipment | Est. Start Date | Est. Total Time to Completion |
| --- | --- | --- |
| Feb-2018 | Q1-2018 | 17 Wks |

The estimated time to completion is considered a low risk to supporting the planned initial shipment. It is low risk as the test setup designed during the Initial Concept / Feasibility stage will be suitable to test the MSV’s. These are non-critical clean parts; therefore, a TMD can be written to test the valves temporarily using the feasibility test setup. This is also the introduction of a new test into an existing process. The current process can also be used to support initial shipment.

###### REQUIRED INPUTS

  - This project will require certain key deliverables, provided in a timely manner, in order to accomplish this plan within the projected schedule.

PRF	-Performance Specification

CEA	-Funding Availability

###### ESTIMATED BUDGET AND HARDWARE REQUIREMENTS:

| Qty | UM | Item Description | Project Phase | Cost Ea | Cost $ |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Micro-BSM | PP | $13,350.28 | $13,350.28 |
| 1 | 1 | Valves, Boards, Flow Meter, Transducer, Misc. Electronic Components | PP | $5,0000.00 | $5,000.00 |
|  |  | Total Project |  |  | $18,350.28. |

###### ABBREVIATIONS AND DEFINITIONS

| Term | Explanation |
| --- | --- |
| ICT | In Circuit Test |
| FT | Functional Test |
| TE | Test Engineering or Test Equipment |
| PCB | Printed Circuit Board |
| RDE | Research and Development Engineering |
| NPI | New Products Introduction |
| CEA | Capital Expenditure Authorization |
| MeOH | Methanol |
| UUT | Unit Under Test |
| PP | Pre-Production |
| RG | Release General |
| PM | Preventative Maintenance |
| MRD | Marketing Requirements Document |
| HW | Hardware |
| SW | Software |

###### RISKS AND CONTINGENCIES:

###### Scope – Significant changes in project scope will result in changes of this plan, which may delay product release. This may include new features or functionality or significant design changes.

###### Test Equipment – Should any of the required test equipment fail qualification; the product will be delayed until the test equipment passes.

###### Hardware/Software Availability – The time scales indicated assumes RDE approved test units will be delivered as required in the various stages and remain available during the development of the test equipment.

###### Resources – Since resources are shared and not necessarily dedicated, conflicts are expected and will be addressed as they occur. Resources shall be fully familiarized with the associated hardware/software options prior to qualification commencing.

  - Budget – The cost of the test equipment is an estimate. The accuracy may vary during development of the test equipment.

  - Schedule – To achieve all TE goals prior to a planned release date, it is imperative TE receives test units early on in RDE’s design phase.

Note: The schedule outlined here assumes no major issues arise during TE development.  Should a failure (or failures) occur, that require a design change to one or more components, the planned schedule will increase by some number of days jeopardizing the project schedule.

###### PROJECT DELIVERABLES:

###### The completion of this project will produce the following deliverables:

  - Test Equipment Design Plan

  - Test Specification and Procedure released to RG

  - Calibration/Maintenance Procedure released to RG

  - Test Station and all related equipment Calibrated/PM’d

  - Test Station Qualified

  - Qualification Documents per procedure 730001835

  - TE Design History Record completed and backed up

- TEST SPECIFICATIONS (TRACEABILITY MATRIX)

The information in this table is subject to change during the development of the product and/or the test equipment. This table will be used to help estimated schedules and cost to develop test equipment. The controlled version of this table will be captured in the released Test Specification and Procedure (TSP).

| Item No. | Manufacturing Test Specification & Condition | Acceptance criteria | Reference |
| --- | --- | --- | --- |
| 1 | MSV Leak Rate – All Ports | USL = 2 uL/min @ 210psi | 735004417 Sec 5.2 |

Add rows as necessary.

- APPENDICES AND REFERENCES:

N/A

END OF DOCUMENT